PYRA

Forest-Floor Wildfire Intelligence Platform

Project Brief  •  Version 0.1  •  Initial Brief for Development Agent

# 1. Mission

PYRA is a ruggedized, stake-shaped sensor node driven into forest ground to continuously measure the physical conditions that govern wildfire ignition and spread. Each node streams data to a cloud platform that fuses on-device sensing with satellite, terrain, weather, and vegetation datasets to produce live fire-danger intelligence for two audiences:

- Operational: firefighters, fire agencies, incident commanders, and dispatchers who need localized, real-time situational awareness.

- Scientific / managerial: forest managers, ecologists, and researchers who need long-term data on fuel conditions, forest dynamics, and climate change impacts on fire regimes.

PYRA is not a new fire-danger model. It is a sensing-and-data platform that feeds established fire-science frameworks (Canadian FWI/FBP, US NFDRS2016, Nelson fuel-moisture models) with better, denser, ground-truth data than any current system.

# 2. Design Principles

- Measure, don't invent. Whenever an established fire-science model exists, we compute it — we do not replace it.

- Sensor fusion over single signals. No single sensor tells the truth; combinations do.

- Graceful degradation. A node that loses comms should still store data locally. A node that loses power should still flash a last-gasp alert. A network that loses half its nodes should still be useful.

- Dual-use from day one. The same raw data feeds firefighter dashboards and research pipelines — different consumers, same source of truth.

- Open interfaces. Use open standards (OGC SensorThings, STAC, GeoJSON, MQTT) so PYRA data can flow into existing agency GIS tools.

- Never create false confidence. Every derived value ships with an uncertainty estimate. Firefighters make life-and-death decisions; they must be able to trust — and appropriately distrust — our outputs.

- Expendable where necessary. Nodes in a fire's path will be lost. Design the unit cost and the data pipeline around that reality.

# 3. System Architecture

A one-page view of the platform, from forest floor to consumer app:

FIELD LAYER

PYRA Node  ─┐

PYRA Node  ─┼── LoRaWAN / BLE mesh ──▶  PYRA Gateway ──▶  encrypted uplink

PYRA Node  ─┘                           (LoRaWAN + cellular/satellite)

CLOUD LAYER

Ingest  ──▶  Time-series DB (Timescale)  ──▶  GIS / PostGIS

+ Fire-Science Engine

(FWI, FBP, Nelson, NFDRS)

Alerting  ◀──┘           Fleet Mgmt             OTA

External feeds: Sentinel-2, DEM, NOAA/ECCC, FIRMS, OSM, CNFDB, LANDFIRE,

lightning networks

CONSUMER LAYER

Firefighter mobile app (iOS/Android)

Research / management web dashboard

Agency GIS plug-ins  (ArcGIS / QGIS)

Public API + data catalogue

# 4. Hardware — The PYRA Node

## 4.1  Form factor

- Stake geometry. ~15 cm conical stainless-steel ground spike, ~40 cm above-ground tube housing electronics, ~25 cm mast for wind sensor / solar panel. Total height ~80 cm deployed.

- Ground coupling. Soil-contact sensors on the buried spike; above-ground sensors in the tube; wind and sun on the mast.

- Enclosure. IP67 minimum, UV-stable polymer + stainless hardware, powder-coated safety-orange for visibility to ground crews. Ceramic-lined upper cap to extend heat survival. Will not survive direct crown fire — don't pretend otherwise.

- Deployment methods: hand-deployed (primary), drone-dropped, vehicle- or helicopter-launched for rapid pre-positioning.

## 4.2  Core electronics

| Component | Suggested part | Rationale |
| --- | --- | --- |
| MCU | ESP32-S3 + LoRa SX1262 companion, or single-chip nRF9160 for cellular | Mature toolchain, cheap, strong community |
| Storage | 8–32 MB flash + microSD slot | Local buffer for days of data during comms outage |
| GNSS | u-blox MAX-M10S | Low-power, multi-constellation |
| RTC | DS3231 w/ coin cell | Accurate timestamps during sleep |
| Security | ATECC608B secure element | Key storage, device attestation, tamper evidence |

## 4.3  Sensor suite

| Parameter | Sensor / technique | Notes |
| --- | --- | --- |
| Soil moisture (2 depths) | Capacitive probe (Teros 10 class) | Proxy for drought stress; calibrate per soil type |
| Soil temperature (2 depths) | DS18B20 on buried cable | Pairs with moisture for energy balance |
| Duff moisture (surface) | Fuel-stick replica + capacitive read | Approximates FFMC inputs |
| Air temperature + RH | SHT45 | Core input to FWI and fuel-moisture models |
| Barometric pressure | BMP390 | Weather context, altitude check |
| Wind speed + direction | Ultrasonic anemometer (FT205) | No moving parts; critical for ISI / FBP |
| Solar irradiance | Silicon pyranometer (Apogee SP-110) | Drives fuel drying, evapotranspiration |
| UV index | SI1133 / LTR390 | Cheap supplement to pyranometer |
| Rain / leaf wetness | Tipping-bucket + capacitive pad | Rainfall is the dominant FFMC driver |
| Smoke / particulates (PM2.5/10) | Plantower PMS7003 or Sensirion SPS30 | Early fire signal; air-quality value |
| Gas: CO, CO₂, VOC | MQ-series + CCS811 / SGP41 | High false-positive — fuse with PM + thermal |
| Thermopile / IR hotspot | Melexis MLX90640 (32×24 IR array) | Nearby heat anomaly + canopy temp |
| Lightning | AS3935 Franklin detector | Ignition-cause tracking; storm warning |
| Acoustic | MEMS mic + TinyML classifier | Chainsaw, vehicle, gunshot, fire-crackle, wildlife |
| PIR motion | Low-power passive IR | Large-mammal + human presence (fuse with acoustic) |
| Accelerometer + magnetometer | ICM-20948 | Slope/aspect at install, tamper/knockdown |
| Camera (higher SKU) | OV5640 + Himax WE-I Plus NPU | Fuel density, ladder fuels, snow, canopy closure |

Not everything ships in every SKU — see §10 Product Tiers.

## 4.4  Power

- Solar: 5–10 W panel, MPPT charge controller (CN3791 or similar).

- Battery: LiFePO₄ 18650 pack — safer thermal behaviour than Li-ion in forest environments, wider temperature range.

- Budget target: survive 14 consecutive days of full canopy shade / cloud cover.

- Power modes: deep sleep default; wake on schedule + interrupt (lightning, accelerometer shock, thermopile threshold, rain start).

- Action item: measure average current draw per subsystem empirically and build a power-budget spreadsheet before designing the PCB.

## 4.5  Deployment-side niceties

- QR code on each node, tying physical unit to cloud record.

- NFC tag for firefighter to tap with phone and pull last 24 h of data offline.

- E-ink display (1.5") confirms successful install: node ID, GPS lock, radio signal, battery.

- Hall-effect "arm" sensor: magnet on a lanyard activates the node; eliminates shelf/shipping drain.

- Capacitive touch / button for manual alert or self-test.

# 5. Communications

## 5.1  Decision matrix

| Link | Range | Power | Bandwidth | Good for |
| --- | --- | --- | --- | --- |
| BLE mesh (node↔node) | ~50 m | Very low | Low | Close-in redundancy, firefighter phone link |
| LoRaWAN | 2–15 km LOS | Low | Very low | Primary backhaul in populated-ish forest |
| LTE-M / NB-IoT | Carrier-dep. | Moderate | Low-mid | Where cellular exists |
| Satellite (Swarm, Iridium SBD, Astrocast) | Global | Moderate | Very low | Backcountry; the killer feature for firefighting |
| Wi-Fi (gateway only) | 50–200 m | High | High | Gateway uplink / maintenance |

## 5.2  Topology

- Node → Gateway → Internet is the primary path (LoRaWAN).

- Gateway sits in a tower, ridge, or mobile command post. One gateway supports 100+ nodes in a 5–10 km radius.

- Satellite-equipped super-nodes: every Nth node carries satellite backup so isolated clusters still report critical alerts.

- BLE local-dump fallback: firefighter walking near any node pulls buffered data via phone even with no network.

## 5.3  Protocol & payload

- Uplink: CBOR-encoded payloads over MQTT (via ChirpStack LoRaWAN NS) or MQTT-SN for cellular.

- Schema: publish a versioned schema; include schema version in every payload.

- Three message classes: Heartbeat (lossy OK); Telemetry (retry with backoff); Alert (immediate, duplicated across radios).

## 5.4  Security

- TLS on every hop that supports it.

- ATECC608B device keys; mutual auth with network server.

- Signed OTA updates; mandatory rollback slot.

- Rate-limit alerts to prevent a compromised node from DoS-ing the dispatch channel.

# 6. Software Stack

## 6.1  Firmware (node)

- Language: C/C++ via ESP-IDF or Zephyr RTOS (Zephyr for MCU portability).

- Modules: sensor drivers, sensor-fusion layer, power manager, comms stack, local alert engine, OTA agent, log store.

- Edge ML: TFLite-Micro for acoustic classifier and optional vision. Models under 512 KB, int8 quantized.

- Local alert engine: runs simple thresholds with no network — e.g. thermopile > 80 °C AND PM2.5 rising AND RH falling → local alarm + store-and-forward.

## 6.2  Cloud backend

- Ingest: MQTT broker (EMQX / Mosquitto) behind a TLS-terminating proxy.

- Stream processor: lightweight Rust/Go worker that validates, decodes, and routes.

- Storage: TimescaleDB (SQL + time-series + PostGIS in one) or InfluxDB; S3-compatible blob store for images/audio/firmware.

- Fire-science engine: scheduled + event-driven worker computing FWI/FBP/NFDRS/Nelson per node on each update, with provenance.

- External data pipeline: daily/weekly ETL for satellite, weather, lightning, OSM — cached and attached to each node's spatial record.

- Alert engine: rule DSL writable by agency users; escalation tiers (watch/warning/emergency); pluggable notifications (SMS, email, Slack/Teams, radio-bridge tone, mobile push).

- Fleet management: inventory, health dashboard, OTA rollout control, recall/decommission workflow.

## 6.3  Consumer apps

- Firefighter mobile app (React Native): map-first, offline tiles, node status + derived index, install workflow, BLE local-dump, incident overlay with wind + slope + predicted spread.

- Research / management web app (React + MapLibre): historical query, CSV/GeoJSON/NetCDF export, model validation tools.

- Agency GIS plug-ins: ArcGIS Pro toolbox and QGIS plug-in consuming the SensorThings API.

- Public/open data portal: subset released under an appropriate licence (see §13).

## 6.4  APIs

- OGC SensorThings API for sensor observations (standard in environmental monitoring).

- STAC catalogue for satellite/raster derivatives.

- REST + GeoJSON for map clients.

- Webhooks for alert subscribers.

- IRWIN adapter (Integrated Reporting of Wildland-Fire Information) for US fire agencies.

# 7. External Data Integrations

Build adapters, not one-offs. Each integration should be a pluggable module with caching, rate-limiting, and provenance logging.

| Dataset | Source | Use |
| --- | --- | --- |
| Digital Elevation Model | USGS 3DEP (US) / CDEM (Canada) / Copernicus GLO-30 | Slope, aspect, sun exposure, spread modeling |
| Sentinel-2 / Landsat | Copernicus / USGS | NDVI, EVI, NBR, NDMI (live fuel moisture proxy) |
| MODIS / VIIRS FIRMS | NASA | Nearby active fires |
| Hansen Global Forest Change | UMD / Google | Forest age, disturbance history |
| LANDFIRE / Canadian NFIS | USGS / NRCan | Fuel-type classification |
| Fire-perimeter history | NIFC, CNFDB, EFFIS | Burn history, repeat-burn risk |
| Weather forecast | NWS / ECCC GeoMet / Open-Meteo | Derived fire-danger forecasting |
| Lightning | Vaisala NLDN / CLDN / Earth Networks | Ignition cause + storm warning |
| Snow cover | MODIS MOD10 / Sentinel-1 SAR | Start of fire season |
| Roads / rivers / structures | OpenStreetMap / cadastre | Natural + man-made barriers, access |
| Drought indices | NOAA CPC / Canadian Drought Monitor | Regional context |
| Air quality | AirNow / ECCC AQHI | Context for PM readings |

# 8. Fire-Science Frameworks (compute, don't invent)

These are the models PYRA data feeds into. Implement each as a versioned library module with tests against published reference cases.

- Canadian Forest Fire Weather Index (FWI) System — FFMC, DMC, DC, ISI, BUI, FWI. Especially important in Quebec; aligns with SOPFEU operations.

- Canadian Fire Behaviour Prediction (FBP) System — FWI + fuel type + slope → rate of spread, fire intensity, crown fire likelihood.

- US NFDRS2016 — cross-border compatibility.

- Nelson dead-fuel moisture model — 1-hr, 10-hr, 100-hr, 1000-hr fuel moistures from weather time series.

- Fosberg Fire Weather Index — instantaneous, widely understood.

- Haines Index — atmospheric instability contribution to plume-dominated fires.

- KBDI (Keetch-Byram Drought Index) — cumulative drought proxy.

Every output carries: model name + version (e.g. `fwi_cfs_2019`); input provenance (sensors + feeds that contributed); uncertainty estimate where derivable.

# 9. What the Node Cannot Sense (and How We Fill It)

A buried stake cannot directly measure several items on the original wish-list. Here's how we get them anyway:

| Wish-list item | How we get it |
| --- | --- |
| Forest age | Hansen Global Forest Change + national forest inventory (per-pixel lookup) |
| Forest density / canopy closure | Sentinel-2 NDVI/LAI + LiDAR where available + install-time photo |
| Species / type of trees | LANDFIRE / NFIS classification + optional install-time tagging |
| Ladder fuels, dead-and-down | Install-time photo + ML classifier in deployment app |
| Human presence | Acoustic (chainsaw/vehicle) + PIR + proximity to OSM roads/trails |
| Natural barriers | OSM + hydrography layers |
| Snow state | MODIS snow cover + node air temp + optional camera |
| "Old bush with dead interior" | Install-time photo → vision model scoring fuel-load severity |

The install-time app workflow is therefore a core product surface, not a nice-to-have.

# 10. Product Tiers (SKUs)

Three tiers let agencies mix-and-match based on strategic value of each site:

| Tier | What's in it | Target price |
| --- | --- | --- |
| PYRA Scout | Core weather + soil + smoke + thermopile + LoRaWAN | Low (dense deployment, semi-expendable) |
| PYRA Sentinel | Scout + anemometer + lightning + gas suite + satellite backup | Mid |
| PYRA Vista | Sentinel + camera + edge ML + higher-res IR array | High (hero nodes at strategic points) |

Gateways are a separate SKU: **PYRA Relay** (solar-powered LoRaWAN gateway with cellular + satellite uplink).

# 11. MVP Roadmap

### Phase 0 — Paper & bench  (weeks 0–6)

Power-budget spreadsheet. Sensor BOM with two candidate parts per slot. Comms decision locked per target geography. Regulatory gap analysis.

### Phase 1 — Single-node proof  (weeks 6–16)

- One hand-soldered node: ESP32-S3 + SHT45 + BMP390 + capacitive soil probe + ultrasonic anemometer + PMS7003 + LoRa.

- ChirpStack + TimescaleDB + PostGIS on a single VM.

- Canadian FWI computed and graphed over a week of data.

- Web map showing the one stake.

### Phase 2 — Small fleet + external data  (weeks 16–28)

- 10 nodes + 1 gateway in a pilot forest (partner with SOPFEU or a university research forest).

- DEM + Sentinel-2 + weather forecast integrations live.

- OGC SensorThings API published.

- Install-time mobile app (basic: photo, GPS, fuel tagging, commissioning check).

### Phase 3 — Operational pilot  (weeks 28–52)

- Satellite-backed super-nodes.

- Alerting with escalation rules.

- Firefighter mobile app v1 with offline maps.

- OTA update pipeline.

- First operational trial with a fire agency partner.

### Phase 4 — Scale  (year 2)

- Edge ML (vision + acoustic).

- Drone-droppable form factor.

- Research-grade data products (monthly QA/QC'd open datasets).

- Integration with IRWIN, ArcGIS, etc.

# 12. Risks & Mitigations

| Risk | Mitigation |
| --- | --- |
| False positives erode firefighter trust | Multi-sensor fusion; tiered alerts; uncertainty estimates; human-in-the-loop for operational alerts |
| Sensor drift (esp. soil moisture, gas) | Seasonal calibration protocol; anomaly detection vs. neighbour nodes |
| Node destruction in fires | Design unit economics assuming % loss per season; cheap Scout tier; satellite super-nodes survive longer |
| Wildlife / vandalism / theft | Tamper detection via accelerometer; safety-orange + "Scientific Equipment" label; GPS theft alerts |
| Comms blackout at the critical moment | BLE local dump; satellite backup on super-nodes; store-and-forward |
| Privacy (cameras, acoustic, humans) | On-device classification only; no raw audio/video leaves the node; privacy-by-design review; clear public policy |
| Data sovereignty (Canadian / provincial) | Canadian-hosted cloud option (AWS ca-central-1 / Azure Canada / OVH Beauharnois); ISED compliance audit |
| Regulatory delays on radio certification | Use pre-certified LoRa/cellular modules; budget for FCC/ISED testing early |
| Firefighter trusts PYRA alone — bad outcomes | UX + training position PYRA as decision support, never sole authority; always display uncertainty |

# 13. Regulatory, Standards & Compliance

- Radio certification: FCC Part 15 (US), ISED RSS-247/210 (Canada), CE RED (EU).

- Safety-of-use near firefighters: review against NFPA standards (e.g. 1977 PPE-compatibility of markings, 1500 programme compatibility) and CAN/ULC equivalents.

- Battery transport: UN 38.3 for LiFePO₄ cells.

- Data standards: OGC SensorThings, OGC API – Features, STAC, NetCDF-CF for research export.

- Open licences: CC-BY-4.0 for public-facing derived datasets; separate commercial licence for real-time feeds.

- Privacy: PIPEDA (Canada), Quebec Law 25, and equivalents per deployment region. Data-protection impact assessments before any camera/acoustic SKU ships.

# 14. Operational & Lifecycle Considerations

- Deployment planning. GIS tool suggesting stake locations from terrain, fuel type, and access — avoid dense clustering; target ridge-lines, saddles, fuel-type boundaries, historical ignition hotspots.

- Field maintenance. Batteries + desiccants replaceable without tools; OTA firmware; mobile-app-driven calibration routines.

- End-of-life. Return-and-recycle programme; lithium cells through partner; polymer selected for recyclability.

- Training. Half-day deployment course with e-learning + printed field card; materials version-controlled alongside firmware.

- Chain of custody. Every observation signed with device key; immutable audit log (important for post-fire investigations).

# 15. Integration Targets

- SOPFEU (Société de protection des forêts contre le feu — Quebec) — natural first partner.

- Parks Canada and provincial fire agencies.

- USFS / CAL FIRE / NIFC / IRWIN (US).

- ALERTCalifornia / ALERTWildfire camera networks — overlay PYRA on the same maps.

- WIFIRE Lab (UCSD) — research collaboration on fire-spread modeling.

- NIST Public Safety Communications Research — interoperability testing.

- Copernicus Emergency Management Service (EU).

# 16. Open Questions for the Agent

The agent should surface these to the human team early rather than guess:

- Primary geography for pilot — Quebec (SOPFEU partnership) or US Pacific Northwest?

- Bundle our own gateway hardware, or certify against existing LoRaWAN gateways?

- Unit-cost ceiling for the Scout tier (drives sensor selection + radio choice).

- Open-source the firmware? The mobile app? The models? Which layers stay proprietary?

- Business model — hardware sale + SaaS? Data-as-a-service? Grant-funded non-profit? Public-private?

- Camera SKU in v1 — privacy complexity vs. enormous analytic payoff?

- Any mandated IRWIN / CAD integrations from a launch-customer agency?

# 17. Appendix A — Suggested Libraries & Tooling

- Firmware: ESP-IDF, Zephyr RTOS, TFLite-Micro, CBOR-C, LoRaMAC-node.

- Backend: Rust (ingest), Python (analytics), TimescaleDB, PostGIS, ChirpStack, EMQX, Apache Airflow, DuckDB.

- Frontend: React, React Native, MapLibre GL, deck.gl, Tailwind.

- Geospatial: GDAL, rasterio, xarray, pystac, rio-tiler, Titiler, QGIS, h3-py.

- Fire-science: cffdrs (R, CFS reference FWI/FBP — port to Python/Rust as needed), firebehaviour Python packages, USFS NFDRS2016 reference code.

- ML: PyTorch → ONNX → TFLite-Micro; Label Studio for labelling.

- DevOps: Terraform, Docker, GitHub Actions, Memfault (or homegrown OTA).

# 18. Appendix B — Naming & Branding

- Product name: PYRA (from pyre / pyros — Greek for fire). Agent must verify TM/USPTO/CIPO clearance before any public launch.

- Tier names: Scout, Sentinel, Vista. Gateway: Relay.

- Tone: tools-for-serious-work. Not gamified, not hype. Firefighters and researchers both respect understated, precise language.

- Visual identity: safety-orange accent, muted greys, topographic-contour motif. Avoid stock flame imagery.

# 19. First Instructions to the Agent

When you begin work, in this order:

- Clarify the seven open questions in §16 with the human lead before spending engineering budget.

- Build the power-budget spreadsheet and the sensor BOM with two candidates per slot. Share for review.

- Stand up the Phase 1 single-node stack end-to-end on a bench — one stake, one laptop, one MQTT broker, FWI computed and graphed.

- Do NOT build a new fire-danger model. Port or wrap cffdrs. If you think you've found an improvement, document it as a research hypothesis, not a product change.

- Every output must carry provenance and uncertainty. No naked numbers.

- Prefer open standards (SensorThings, STAC, GeoJSON, MQTT) over bespoke APIs. An agency that cannot ingest our data will not buy our platform.

- Keep a running DECISIONS.md — every non-trivial design choice, who made it, and why. This becomes the audit trail.

End of brief. Iterate freely — this is v0.1.